# CTS MDC—FAS # OCCUPATIONAL HAZARD ASSESSMENT AND CONTROL

Note 1: Alberta’s Occupational Health and Safety Code (OH&S Code) provides specific compliance requirements. This document is meant as a summary or checklists to assist staff comply with the legislation or applicable school jurisdiction administrative policies and regulations.

Note 2: The employee is required to conduct a pre-job hazard assessment before commencing work.

Note 3: This is a general worksite hazard assessment. Depending on the school/instructional area/specific courses taught, some incidents/hazards may or may not be present.

| RISK ANALYSIS (A x B x C = DEGREE OF RISK) |  |  |  |  |
| --- | --- | --- | --- | --- |
| A Frequency of Exposure to Hazard | B Incident/Hazard Probability | C Potential Consequence | Degree of Risk | Risk Classification |
| 1 = Less than once a month 2 = At least once a month 3 = At least once a week 4 = One or more times daily | 1 = Not likely 2 = Remote: not likely by once every 5–20 yrs 3 = Occasional: likely to happen once every 1–5 yrs 4 = Probable: expected to happen one or more times/year | 1 = Negligible (no injury, first aid) 2 = Marginal (medical aid, minor injury or illness) 3 = Critical (lost time injury, temp. disability) 4 = Catastrophic (serious injury or illness) | 1–11 12–31 32–64 | L – Low M – Medium H – High |

| POTENTIAL HAZARD | POTENTIAL HAZARD DESCRIPTION | RISK ANALYSIS (A x B x C = Degree of Risk) | SUMMARY OF EXISTING CONTROLS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ENGINEERING | ADMINISTRATIVE Alberta’s Occupational Health and Safety Codes (OH&S Code) applicable school jurisdiction administrative policies and regulations | PERSONAL PROTECTIVE EQUIPMENT (PPE) Personal Protective Equipment (PPE) appropriate for the work is both worn and CSA certified/approved where required. | CONTROL IN PLACE YES/NO |
| Biological Exposure | - body fluid exposure from injuries | Biological 4 x 4 x 1 = 16 Medium | - sink for handwashing - first aid kit and supplies | - Use universal precautions. - Ensure that personnel are trained in first aid. - Post emergency procedures. - Identify and establish sanitary and safe housekeeping practices. | - protective gloves (latex or nitrile disposable gloves) - safety glasses, goggles |  YES  NO |
|  | - close contact with students and others with illnesses | Biological 4 x 4 x 2 = 32 High | - n/a | - Establish handwashing practices. - Follow universal precautions. - Provide hand sanitizer in rooms without sinks. | - n/a |  YES  NO |
| Chemical Exposure | - exposure to cleaning agents, white board markers and cleaner, sanitizers | Chemical 4 x 2 x 1 = 8 Low | - n/a | - Ensure WHMIS training is available. - Provide up-to-date MSDS. - Follow safe handling and storage information on labels or MSDS. - Clean the white board with a microfibre cloth and water (no chemicals). | - n/a |  YES  NO |
|  | - poor indoor air quality (lack of fresh air, stale and stuffy) |  | - general ventilation | - Confirm that the ventilation system is on and there is air movement. | - n/a |  YES  NO |
| Ergonomic Exposure | - improper workstation setup for teacher - repeated awkward movements over a prolonged period | Ergonomic 4 x 4 x 2 = 32 High | - n/a | - Use a proper workstation setup. - Ensure repetitive tasks are properly paced. - Take frequent breaks. | - n/a |  YES  NO |
|  | - lifting and moving furniture, materials, and equipment | Ergonomic 4 x 4 x 2 = 32 High | - n/a | - Perform a hazard assessment, considering the weight, size, shape, and number of times and manner in which the load will be moved. | - n/a |  YES  NO |
| Physical Exposure | - damaged electrical connections, outlets, extension cords, equipment | Physical 4 x 4 x 3 = 48 High | - n/a | - Check condition of electrical connections and cords for damage. Repair or replace damaged items. | - n/a |  YES  NO |
|  | - trip hazard due to extension cords, A/V cords, etc. | Physical 4 x 4 x 3 = 48 High | - n/a | - Manage cords and cables to reduce trip hazards (e.g., route under area rugs). - Good housekeeping practices to reduce trip hazards. Clean up spills. | - n/a |  YES  NO |
|  | - damaged furniture - filing cabinets drawers left open | Physical 4 x 3 x 2 = 24 Medium | - n/a | - Replace, repair or remove damaged furniture. - Keep furniture doors closed when not in use. | - n/a |  YES  NO |
|  | - falls from heights | Physical 4 x 4 x 3 = 48 High | - n/a | - Do not use chairs and furniture as substitutes for a stepladder when accessing bulletin board. - Use a step ladder when working at heights. | - n/a |  YES  NO |
|  | - falling equipment | Physical (working at heights) 2 x 3 x 3 = 18 Medium | - n/a | - Follow manufacturer’s instructions when operating machinery. - Periodically check overhead projector screens connections or brackets to ensure they are secured, and will not fall if opened. | - n/a |  YES  NO |
| Psycho-social Exposure | - bullying and verbal harassment with adults and students | Psychosocial 1 x 4 x 1 = 4 Low | - n/a | - Follow visitor procedures. - Follow working alone procedures for school staff (including security and door locking procedures). - Follow safe and secure school procedures. | - n/a |  YES  NO |

© Calgary Board of Education