# CTS TMT—CON # OCCUPATIONAL HAZARD ASSESSMENT AND CONTROL

Note 1: Alberta’s Occupational Health and Safety Code (OH&S Code) provides specific compliance requirements. This document is meant as a summary or checklists to assist staff comply with the legislation or applicable school jurisdiction administrative policies and regulations.

Note 2: The employee is required to conduct a pre-job hazard assessment before commencing work.

Note 3: This is a general worksite hazard assessment. Depending on the school/instructional area/specific courses taught, some incidents/hazards may or may not be present.

| RISK ANALYSIS (A x B x C = DEGREE OF RISK) |  |  |  |  |
| --- | --- | --- | --- | --- |
| A Frequency of Exposure to Hazard | B Incident/Hazard Probability | C Potential Consequence | Degree of Risk | Risk Classification |
| 1 = Less than once a month 2 = At least once a month 3 = At least once a week 4 = One or more times daily | 1 = Not likely 2 = Remote: not likely by once every 5–20 yrs 3 = Occasional: likely to happen once every 1–5 yrs 4 = Probable: expected to happen one or more times/year | 1 = Negligible (no injury, first aid) 2 = Marginal (medical aid, minor injury or illness) 3 = Critical (lost time injury, temp. disability) 4 = Catastrophic (serious injury or illness) | 1–11 12–31 32–64 | L – Low M – Medium H – High |

| POTENTIAL HAZARD | POTENTIAL HAZARD DESCRIPTION | RISK ANALYSIS (A x B x C = Degree of Risk) | SUMMARY OF EXISTING CONTROLS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ENGINEERING | ADMINISTRATIVE Alberta’s Occupational Health and Safety Codes (OH&S Code) applicable school jurisdiction administrative policies and regulations | PERSONAL PROTECTIVE EQUIPMENT (PPE) Personal Protective Equipment (PPE) appropriate for the work is both worn and CSA certified/approved where required. | CONTROL IN PLACE YES/NO |
| Biological Exposure | - body fluid exposure from injuries | Biological 4 x 4 x 1 = 16 Medium | - sink for handwashing - first aid kit and supplies | - Use universal precautions. - Ensure that personnel are trained in first aid. - Post emergency procedures. - Identify and establish sanitary and safe housekeeping practices. - Establish handwashing practices. | - protective gloves (latex or nitrile disposable gloves) - safety glasses, goggles |  YES  NO |
| Chemical Exposure | - contact with finishes (paints) and solvents that may be splashed into eyes, absorbed into the skin, or inhaled. | Chemical 4 x 4 x 2 = 32 High | - local and general ventilation | - Ensure WHMIS training is available. - Provide up-to-date MSDS. - Follow safe handling and storage information on labels or MSDS. - Ensure regular cleaning and maintenance of ventilation system. | - protective gloves - eye protection - apron |  YES  NO |
|  | - respiratory hazard and potential allergic response to dust and particulate from processing of wood materials, and toxic woods (e.g., cedar) | Chemical 4 x 4 x 2 = 32 High | - local and general ventilation - dust collection | - Identify and establish sanitary and safe housekeeping practices. - Secure and use the least toxic and harmful substance for the students and the environment. | - approved half respirators for toxic dust |  YES  NO |
|  | - dust from wood as a flammable particulate | Chemical 4 x 4 x 2 = 32 High | - dust collection systems | - Identify and establish sanitary and safe housekeeping practices. - Avoid machining wood with nails/staples that could cause sparks. - Avoid using the dust collection system to clean up metallic debris, such as nails and staples, as they may cause sparks a source of ignition. - Do not use compressed air to clean clothing or machinery. - Follow the manufacturer’s instructions for operation, maintenance, and repair of tools and equipment. | - n/a |  YES  NO |
|  | - flammable finishes (spray and brushed) | Chemical 4 x 4 x 2 = 32 High | - flammable finishes applied in finishing room where there is local and general exhaust ventilation - flammable finishes spray applied in room with intrinsically safe electric equipment - flammable liquids dispensed in area with local exhaust ventilation - flammable liquids stored in non-flammable storage cabinets | - Ensure WHMIS training is available. - Provide up-to-date MSDS. - Follow safe handling and storage information on labels or MSDS. - Dispose of flammable waste in appropriate waste containers, which are cleaned out at regular intervals and not allowed to accumulate. | - n/a |  YES  NO |
| Ergonomic Exposure | - lifting or moving heavy and awkward materials | Ergonomic 4 x 4 x 2 = 32 High | - n/a | - Perform a hazard assessment, considering the weight, size, shape, and number of times and manner in which the load will be moved. | - n/a |  YES  NO |
| Physical Exposure | - confined space (e.g., entering dust collector to check condition of bags) | Physical 4 x 4 x 3 = 48 High | - n/a | - Maintain and follow the manufacturer’s manuals for the operation, maintenance, and repair of equipment. - Provide and document ongoing safety education and training, including mandatory equipment and tool orientation. - Follow procedures for working in confined spaces. | - hard hats - safety glasses - safety footwear (construction sites) - hearing protection - approved respirator |  YES  NO |
|  | - shocks and electrocution (e.g., if equipment is not locked out) | Physical 4 x 4 x 3 = 48 High | - n/a | - Maintain and follow the manufacturer’s manuals for the operation, maintenance, and repair of equipment. - Provide and document ongoing safety education and training, including mandatory equipment and tool orientation. | - n/a |  YES  NO |
|  | - contact with moving/flying objects (e.g., wood being cut with a table saw, materials being shaped by lathe) | Physical 4 x 4 x 3 = 48 High | - equipment guards - anti-kickback devices | - Provide and document ongoing safety education and training, including mandatory equipment and tool orientation. - Provide initial training to include a thorough review of hazards and accidents associated with the job. | - n/a |  YES  NO |
|  | - contact with moving parts that can cut, pinch, entangle | Physical 4 x 4 x 3 = 48 High | - equipment guards | - Provide and document ongoing safety education and training, including mandatory equipment and tool orientation. - Provide initial training to include a thorough review of hazards and accidents associated with the job. - Secure loose clothing, tie back long hair, and remove jewellery. | - n/a |  YES  NO |
|  | - contact with other persons within the shop while working at equipment | Physical 4 x 4 x 3 = 48 High | - n/a | - Mark lines on the floor to designate the work space. - Schedule work to avoid overcrowding. | - n/a |  YES  NO |
|  | - contact with persons or objects being moved within the shop | Physical 4 x 4 x 3 = 48 High | - n/a | - Mark lines on the floor to designate work space. - Schedule work to avoid overcrowding. | - n/a |  YES  NO |
|  | - electrical hazards associated with damaged cords, damaged operating controls, or access to service disconnects and breakers | Physical 4 x 4 x 3 = 48 High | - n/a | - Tag and remove defective tools/equipment from service. | - n/a |  YES  NO |
|  | - electrical hazard from servicing equipment that has hazardous energy that has not been locked out/tagged out or brought to “zero” energy state | Physical 4 x 4 x 3 = 48 High | - n/a | - Tag and remove defective tools/equipment from service. | - n/a |  YES  NO |
|  | - fall down steps - fall when accessing dust collector for maintenance | Physical 4 x 4 x 3 = 48 High |  | - Follow good housekeeping practices. - Use handrails, keep steps free and clear of trip and slip hazards, avoid carrying materials that obstruct view of path of travel. - work within work platform on dust collector | - n/a |  YES  NO |
|  | - fire risk resulting from friction (nails or metal in machined wood) | Physical 4 x 4 x 3 = 48 High | - n/a | - Post location of fire extinguishers and evacuation procedures. | - n/a |  YES  NO |
|  | - material stored that may tip or fall and strike a person | Physical 4 x 4 x 3 = 48 High | - n/a | - Keep materials stored in such a manner that they will not tip or fall and strike a person. | - n/a |  YES  NO |
|  | - noise exposure when equipment is operating | Physical 4 x 4 x 3 = 48 High | - n/a | - n/a | - hearing protection |  YES  NO |
|  | - working at heights (roof construction projects) | Physical 4 x 4 x 3 = 48 High | - n/a | - Always have someone holding the ladder. - Have a fall protection plan in place if working at heights greater than 3 m. | - fall protection (anchor point, lanyard, and full body harness) |  YES  NO |
| Psycho-social Exposure | - bullying and verbal harassment with adults and students | Psychosocial 1 x 4 x 1 = 4 Low | - n/a | - Follow visitor procedures. - Follow working alone procedures for school staff (including security and door locking procedures). - Follow safe and secure school procedures. | - n/a |  YES  NO |

© Calgary Board of Education