# CTS TMT—MEC # OCCUPATIONAL HAZARD ASSESSMENT AND CONTROL

Note 1: Alberta’s Occupational Health and Safety Code (OH&S Code) provides specific compliance requirements. This document is meant as a summary or checklists to assist staff comply with the legislation or applicable school jurisdiction administrative policies and regulations.

Note 2: The employee is required to conduct a pre-job hazard assessment before commencing work.

Note 3: This is a general worksite hazard assessment. Depending on the school/instructional area/specific courses taught, some incidents/hazards may or may not be present.

| RISK ANALYSIS (A x B x C = DEGREE OF RISK) |  |  |  |  |
| --- | --- | --- | --- | --- |
| A Frequency of Exposure to Hazard | B Incident/Hazard Probability | C Potential Consequence | Degree of Risk | Risk Classification |
| 1 = Less than once a month 2 = At least once a month 3 = At least once a week 4 = One or more times daily | 1 = Not likely 2 = Remote: not likely by once every 5–20 yrs 3 = Occasional: likely to happen once every 1–5 yrs 4 = Probable: expected to happen one or more times/year | 1 = Negligible (no injury, first aid) 2 = Marginal (medical aid, minor injury or illness) 3 = Critical (lost time injury, temp. disability) 4 = Catastrophic (serious injury or illness) | 1–11 12–31 32–64 | L – Low M – Medium H – High |

| POTENTIAL HAZARD | POTENTIAL HAZARD DESCRIPTION | RISK ANALYSIS (A x B x C = Degree of Risk) | SUMMARY OF EXISTING CONTROLS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ENGINEERING | ADMINISTRATIVE Alberta’s Occupational Health and Safety Codes (OH&S Code) applicable school jurisdiction administrative policies and regulations | PERSONAL PROTECTIVE EQUIPMENT (PPE) Personal Protective Equipment (PPE) appropriate for the work is both worn and CSA certified/approved where required. | CONTROL IN PLACE YES/NO |
| Biological Exposure | - body fluid exposure from injuries | Biological 4 x 4 x 1 = 16 Medium | - sink for handwashing - first aid kit and supplies | - Use universal precautions. - Ensure that personnel are trained in first aid. - Post emergency procedures. - Identify and establish sanitary and safe housekeeping practices. - Establish handwashing practices. | - protective gloves (latex or nitrile disposable gloves) - safety glasses, goggles |  YES  NO |
| Chemical Exposure | - exposure to metal cleaners, battery acid, solvent-based paints and thinners | Chemical 4 x 4 x 3 = 48 High | - eyewash station - equipment in safe and proper working order - ventilation system | - Ensure WHMIS training is available. - Provide up-to-date MSDS. - Follow safe handling and storage information on labels or MSDS. - Ensure regular cleaning and maintenance of ventilation system. - Establish handwashing practices. | - Welding goggles - Welding helmet - Protective gloves and leather apron |  YES  NO |
|  | - exposure to carbon monoxide and other fumes/vapours from vehicle exhaust | Chemical 4 x 4 x 3 = 48 High | - local exhaust ventilation and general ventilation within the shop (i.e., tail pipe connected to local ventilation) | - Ensure WHMIS training is available. - Provide up-to-date MSDS. - Follow safe handling and storage information on labels or MSDS. - Ensure regular cleaning and maintenance of ventilation system. - Minimize vehicle running within shop area. | - n/a |  YES  NO |
|  | - exposure to fumes from welding/brazing | Chemical 4 x 4 x 3 = 48 High | - local ventilation systems in welding areas - portable local exhaust system when welding in general shop area | - Ensure WHMIS training is available. - Provide up-to-date MSDS. - Follow safe handling and storage information on labels or MSDS. - Ensure regular cleaning and maintenance of ventilation system. | - n/a |  YES  NO |
|  | - exposure to dust from grinding and sanding | Chemical 4 x 4 x 3 = 48 High | - local exhaust system attached to hand grinders - eyewash station | - Minimize dry sweeping and, whenever possible, use vacuums with filters to clean up dust. - Ensure regular cleaning and maintenance of ventilation system. | - safety glasses or goggles |  YES  NO |
|  | - exposure to sand blasting (parts cleaner) and cabinet | Chemical 4 x 4 x 3 = 48 High | - eyewash station | - Minimize dry sweeping and, whenever possible, use vacuums with filters to clean up dust. - Ensure regular cleaning and maintenance of ventilation system. | - safety glasses or goggles |  YES  NO |
|  | - exposure to vapours from paints and thinners, fillers | Chemical 4 x 4 x 3 = 48 High | - local and general ventilation - spray booth | - Ensure regular cleaning and maintenance of ventilation system. | - respirator |  YES  NO |
|  | - fire risk from flammable solvent-based paints and thinners | Chemical 4 x 4 x 3 = 48 High | - spray booth ventilation | - Ensure WHMIS training is available. - Provide up-to-date MSDS. - Follow safe handling and storage information on labels or MSDS. - Ensure regular cleaning and maintenance of ventilation system. - Store chemicals in a separate room and in fire-rated cabinets. | - n/a |  YES  NO |
|  | - explosion hazard from compressed gases | Chemical 4 x 4 x 3 = 48 High | - compressed gases stored in trolley/secured to trolley - flashback arrestor and reverse flow check valve - local exhaust ventilation | - Check hoses for leaks and replace damaged or leaking hoses. - Test regulators on cylinders for leaks. - The gas torch is to be used by trained staff only. - Extra cylinders are securely stored in the vertical position with a protective cap covering the valve. - Store chemicals in a separate room and in fire-rated cabinets. | - n/a |  YES  NO |
| Ergonomic Exposure | - lifting and moving awkward and/or heavy parts, equipment, and materials | Ergonomic 4 x 4 x 3 = 48 High | - n/a | - Provide training in proper lifting technique. - Get help with heavy or awkward loads if a lifting aid is not available. - Assess the load; bend your knees; grasp the load with a firm grip; hold the load close to your body; and lift with your legs, keeping your back straight. - Use lifting and positioning aids to accommodate work posture. - Store heavy objects at waist height. - Try to use better designed work tools and workstations. - Use tools with a weight counter-balancer. | - Gloves - safety boots (CSA approved) |  YES  NO |
|  | - working in awkward positions for long periods of time | Ergonomic 4 x 4 x 3 = 48 High | - n/a | - Rotate jobs as frequently as possible. - Avoid working in one position for long periods of time. - Take frequent breaks to stretch. - Minimize awkward postures. | - n/a |  YES  NO |
|  | - injury caused by repetitive motions | Ergonomic 4 x 4 x 3 = 48 High | - n/a | - Learn to recognize repetitive stress injuries. - Ensure repetitive tasks are properly paced. - Take regular breaks. | - n/a |  YES  NO |
| Physical Exposure | - confined space (e.g., autobody spray booth, if cleaning downdraft area below grade) | Physical 4 x 4 x 3 = 48 High | - n/a | - Be familiar with safe operating and maintenance requirements. - Follow proper procedures for working in confined spaces. | - n/a |  YES  NO |
|  | - fire hazard and explosion hazard due to compressed-gas cylinders | Physical 4 x 4 x 3 = 48 High | - reverse-flow check valves to prevent flashback | - Compressed gases cylinders are stored and handled in carts (never on side). - Regulators are tested for leaks. - Store and protected against damage and secure against falling. | - n/a |  YES  NO |
|  | - contact with compressed air that may penetrate skin | Physical 4 x 4 x 3 = 48 High | - nozzle with pressure relief holes | - Do not use compressed air to clean clothing or skin. | - n/a |  YES  NO |
|  | - burns from hot surfaces - metal splashes during welding, cutting - handling hot tools and equipment | Physical 4 x 4 x 3 = 48 High | - n/a | - Maintain good housekeeping. | - welding goggles (CSA approved) - welding helmet - protective gloves and clothing - leather apron |  YES  NO |
|  | - contact with portable and pedestal grinders that are improperly guarded or adjusted | Physical 4 x 4 x 3 = 48 High | - pedestal grinder work rest adjusted properly (3 mm gap max) | - Dress the pedestal grinder wheel regularly. | - face shield |  YES  NO |
|  | - contact with sharp edges and materials, auto parts | Physical 4 x 4 x 3 = 48 High | - n/a | - Cover sharp materials with tape or other protective material. | - n/a |  YES  NO |
|  | - entanglement in rotating machinery, parts, belts, and tools | Physical 4 x 4 x 3 = 48 High | - equipment guards | - Secure loose clothing, tie back long hair, and remove all jewellery. | - protective gloves and clothing - leather apron |  YES  NO |
|  | - pinch points on equipment and/or machinery with V-belt drives (e.g., compressors) | Physical 4 x 4 x 3 = 48 High | - equipment guards | - Secure loose clothing, tie back long hair, and remove all jewellery. | - n/a |  YES  NO |
|  | - electrical shock from damaged or exposed connections, inadequate grounding, welding processes, worn or damaged cables/cords, lack of proper gloves, and working in wet conditions | Physical 4 x 4 x 3 = 48 High | - n/a | - Equipment that is being repaired and/or maintained is locked/tagged out (zero energy state). - Tag and remove defective tools/equipment from use. - Check electrical cords and cables for damage. Repair or replace damaged cords and cables. | - gloves - glasses with side shields - face shield - non-conductive footwear (high voltage) for welding - hearing protection |  YES  NO |
|  | - falling objects, including vehicles | Physical 4 x 4 x 3 = 48 High | - n/a | - Materials or vehicles that are elevated must be secured against falling. - Make sure the hoist is rated to handle the load. - Communicate with others when moving materials. - Use safety stands whenever working underneath a vehicle. | - n/a |  YES  NO |
|  | - noise from grinding and certain types of welding | Physical 4 x 4 x 3 = 48 High | - n/a | - n/a | - hearing protection |  YES  NO |
|  | - fire and explosion from welding or cutting close to combustible materials or welding or cutting gases from poorly fitting and leaky hoses - fire risk from flammable/combustible liquids and wastes (i.e., rags) | Physical 4 x 4 x 3 = 48 High | - local ventilation - fire extinguishers and evacuation procedures | - Use flammable liquids and gases in well-ventilated areas. - Secure and use the least toxic materials for both students and the environment. | - leather gloves - safety glasses - leather apron |  YES  NO |
|  | - slip hazard from vehicle fluids, spilled paints, or water - trip hazards from cables, hoses, and debris | Physical 4 x 4 x 3 = 48 High | - n/a | - Use good housekeeping practices. - Put away all tools, equipment, and materials in their proper storage areas. - Clean up spills immediately. | - n/a |  YES  NO |
|  | - welding arc flash (UV light or infrared radiation) | Physical 4 x 4 x 3 = 48 High | - n/a | - Follow electrical safety procedures. - Ensure cables are in good condition and are properly grounded. - Avoid standing in water or on wet surfaces. - Avoid wearing sweaty clothes. - Turn the disconnect switch off before plugging in or unplugging the high-voltage input cable. - Ensure welding equipment is correctly connected before using. - Ensure that main power terminals are located inside welding machine cover; ensure terminals are accessible only with tools. - Protect cables form damage. - Never touch wire spool while the wire is being fed. | - gloves - glasses with side shields - face shield - non-conductive footwear (high voltage) for welding - hearing protection |  YES  NO |
|  | - fire hazards due to welding | Physical 4 x 4 x 3 = 48 High | - n/a | - Do not cut or use a torch to cut old paint containers/drums or containers that may have contained flammable liquids. | - welding shields and/or welding glasses - helmet - clothing made for welding (i.e., resistant to spark, flame, and heat) - non-conductive footwear |  YES  NO |
|  | - unsafe vehicle (e.g., not road worthy, faulty brakes, leaking oil or gasoline) | Physical 4 x 4 x 3 = 48 High | - n/a | - Perform a circle check on any vehicle that is under power (brakes, leaks, or engine malfunction). - Drive safely. - Communicate movement of vehicles in/out of shop. - Use safety stands. | - n/a |  YES  NO |
|  | - vibration from tools/equipment | Physical 4 x 4 x 3 = 48 High | - vibration isolation - suspension systems | - Rotate jobs as often as possible. - Take frequent rest periods. - Reduce intensity and duration of exposure. - Provide and document ongoing safety education and training, including mandatory equipment and tool orientation. | - n/a |  YES  NO |
|  | - falls from height | Physical 4 x 4 x 3 = 48 High | - n/a | - Use proper ladders when working at heights (three-point contact). | - anchor point, lanyard, and full body harness |  YES  NO |
| Psycho-social Exposure | - bullying and verbal harassment with adults and students | Psychosocial 1 x 4 x 1 = 4 Low | - n/a | - Follow visitor procedures. - Follow working alone procedures for school staff (including security and door locking procedures). - Follow safe and secure school procedures. | - n/a |  YES  NO |

© Calgary Board of Education