| MDC: media, design & communication Arts |  |  |  |
| --- | --- | --- | --- |
| Communications Technology |  |  |  |
| task | Developing a Photo—Developer Process |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - fire and explosion |  | - burns - smoke inhalation |
| Chemical | - flammable and combustible - toxic - corrosive |  | - skin rash - eye irritation - burns - allergic reaction - lung damage, if inhaled |
| Biological | - n/a |  | - n/a |
| Ergonomic | - n/a |  | - n/a |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - illness |
| Pre-operational Safety Checks |  |  |  |
| - Ensure the ventilation system is working. - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. - Inspect required PPE and replace if required. - Report any hazards. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on all required PPE. Use eye, hand, and body protection. Do not let developer make contact with eyes or with skin. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Follow directions. Before you start: - Close darkroom door and turn on safety lights. - Put on an apron and safety goggles. - Wash both sets of tongs. - Replace water in Tray 3. - Developer: - Add developer to Tray 1. - Place the photo in Tray 1 with the shiny side up. - Gently slosh the developer in Tray 1 back and forth for about 1 minute to immerse the photo. - Use tongs to remove the photo from the developer. - Put away all tools, equipment, and materials in their proper storage areas. - Clean the work area. - Establish handwashing practices. |  | - Store prints at or below eye level to prevent splashes to eyes if dropped. - Do not breathe vapors. - Don’t put bare hands into developer baths; use tongs instead. - Cover all baths when not in use. - Situate emergency first aid and eyewash stations close to the darkroom. - Store developer in appropriate designated cabinet. - Developer can cause skin irritation to some people, so avoid contact and wash your hands often if you get the chemical on you. This chemical will stain your clothes when it dries. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |