| MDC: media, design & communication Arts |  |  |  |
| --- | --- | --- | --- |
| Communications Technology |  |  |  |
| task | Developing a Photo |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - fire and explosion |  | - burns - smoke inhalation |
| Chemical | - corrosive: developer, fixer - fumes: developer, fixer - flammable and combustible: developer, fixer |  | - skin rash - eye irritation - burns - allergic reaction - lung damage, if inhaled |
| Biological | - n/a |  | - n/a |
| Ergonomic | - n/a |  | - n/a |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - illness |
| Pre-operational Safety Checks |  |  |  |
| - Ensure ventilation system is working. - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. - Inspect required PPE and replace if required. - Review WHMIS workplace labels. - Report any hazards. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on required PPE. Use eye, hand, and body protection. Do not let chemicals make contact with eyes or with skin. - Secure loose clothing, and tie back long hair. - Follow the school safety policy. - Follow the manufacturer’s instructions. - Follow directions carefully. Before you start: - Close darkroom door and turn on safety lights. - Wash two sets of tongs. - Add water to Tray 3. - Developer: - Place developer in Tray 1. - Place the photo into Tray 1 with the shiny side up. - Gently slosh developer in Tray 1 back and forth for about 1 minute to immerse the photo. - Use tongs to remove photo from Tray 1. - Fixer: - Add fixer to Tray 2. - Using tongs, place photo into Tray 2. Do not put the tongs from the developer into the fixer. - Leave the photo in tray for 2 minutes. - Use a different set of tongs to remove the photo from Tray 2. (Make sure that it is a different set of tongs than those used in the developer bath.) - Water: - Place the photo in Tray 3. - Leave the photo in the tray for 3 minutes. - Rinse tongs used with Tray 2. - Use tongs to remove the photo from Tray 3. - Hang the photo to dry. - When you are done: - Wipe up the sink area. - Cover Tray 1 (developer) and Tray 2 (fixer) with black plastic covers. - Wash both sets of tongs. - Empty water from Tray 3. - Store dry photos and negatives in a folder. - Turn off the lights. - Clean the work area. - Establish handwashing practices. |  | - Store prints at or below eye level to prevent splashes to eyes if dropped. - Do not breathe vapours. - Don't put bare hands into developer baths; use tongs instead. - Cover all baths when not in use. - Situate emergency first aid and eyewash stations close to the darkroom. - Store developer and fixer in appropriate designated cabinet. - Both developer and fixer can cause skin irritation to some people, so avoid contact with both chemicals and wash your hands if you get any chemical on you. This chemical will stain your clothes when it dries. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |