| MDC: media, design & communication Arts |  |  |  |
| --- | --- | --- | --- |
| Communications Technology |  |  |  |
| task | Print Screening |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - electrical - strong light - leaning on glass - fire and explosion - pressure washer system - slips and trips - heat and cold stress |  | - burns - smoke inhalation - electric shock - eye damage - cuts - bruising |
| Chemical | - chemicals: citra paste, chroma wet, plastisol inks, red-coat - flammable and combustible materials: citra paste, chroma wet |  | - eye irritation - skin irritation - burns |
| Biological | - bacteria/viruses - blood - body fluids |  | - illness |
| Ergonomic | - n/a |  | - n/a |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. - Inspect required PPE and replace if required. - Inspect room for ventilation. - Report any hazards. - Review WHMIS workplace labels. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on all required PPE. Use hand, body, and eye protection. Avoid chemical contact with eyes or direct contact with skin. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions for equipment and for chemicals. - Follow the school safety policy. - Wipe up spills immediately. - Clean the screen with citra paste. Rinse using a pressure washer system. - Apply chroma wet to the print surface of the screen using a brush. Rinse thoroughly with clean water. Flood the screen with heavy water flow (not fine spray) for a uniform sheet of water over screen. - Apply the CDF film (light sensitive) sheet. Press out any air bubbles. Place in the drying booth. - Create a black and white image. - Print onto a transparency sheet. - Place the transparency and prepared screen onto the exposure unit. - Apply red-coat. - Apply plastisol inks. - Heat seal. - Do not let your hands come into contact with the platen or the dry mount press. Do not close the press on your hands. - Dry mount with the heat setting prescribed by the teacher. - When closing the platen of the dry mount press, be sure that the print and the mount are the only items under the platen. - Turn off and unplug the press at the end of the process. - Put away all tools, equipment, and materials in their proper storage areas. - Clean the work area. - Establish handwashing practices. |  | - Store chemicals at or below eye level to prevent splashes to eyes if dropped. - Do not breathe vapours. - Avoid exposing chemicals to sparks or flames. - Do not lean on or apply pressure to the glass of exposure unit. - Keep hands out of the spray of the pressure washer system. - Do not point pressure washer at anyone. - Direct the pressure washer away from you. - Close the lid before starting the exposure unit. - Do not touch platen of the dry mount machine. - Never test the temperature of the press by touching with your hand. - Print the screen in a well-ventilated room. - Situate emergency eyewash stations close to print screening area. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |