| CLUSTER | MDC: MEDIA, DESIGN, AND COMMUNICATION ARTS |
| --- | --- |
| OCCUPATION | Fashion |
| HAZARD CONTROLS | EQUIPMENT—COMPUTER AND PRINTER |
| Engineering Controls | Goodshoot/Thinkstock |
| - n/a |  |
| Administrative Controls |  |
| - Provide adequate instruction on lab routines, job procedures, and equipment operation. - Follow the manufacturer’s directions for operation, maintenance, and safe practices. - Keep the work area clean and tidy at all times. - Prevent cords from becoming trip hazards in high-traffic areas. - Tag and remove defective equipment from service. - Arrange the equipment to permit safe, efficient work practices and ease of movement and clean-up. - Plug the cord into computer first, and then plug the cord into the socket. - Keep food and beverages away from the equipment. - Sanitize the keyboard. - Inspect cords and plugs for fraying and exposed wires. - Ensure that correct lighting is available. - Ensure an ergonomic set-up of the equipment and furniture. - Follow the school safety policy. - Provide and document ongoing safety education and training, including mandatory equipment and tool orientation. - Ensure equipment is used under competent supervision. - Ensure repetitive tasks are properly paced. - Post important emergency telephone numbers and contacts. - Post lockdown procedures. - Post fire drill procedures. - Establish handwashing practices. - Ensure fire extinguishers are strategically located and marked. - Provide a first aid kit and supplies, including an eyewash station and biohazard disposal. - Correctly place and maintain smoke detectors and heat sensors. |  |
| Personal Protective Equipment (PPE) |  |
| - wrist rest - adjustable chair |  |