| CLUSTER | MDC: MEDIA, DESIGN, AND COMMUNICATION ARTS |
| --- | --- |
| OCCUPATION | Fashion |
| HAZARD CONTROLS | EQUIPMENT—EMBROIDERY MACHINE |
| Engineering Controls | combination sewing/embroidery machine industrial embroidery machine Clockwise left to right: © byheaven/30896182/Fotolia © 2013 Alberta Education © srki66/42945514/Fotolio © Moreno Soppelsa/shutterstock adapted from © srki66/42945514/Fotolio |
| - n/a |  |
| Administrative Controls |  |
| - Provide adequate instruction on lab routines, job procedures, and equipment operation. - Keep the work area clean and tidy at all times. - Post safety signage. - Follow the manufacturer’s directions for operation, threading, maintenance, and safe practices. - Inspect the machine regularly to ensure safe operation. - Ensure a clear line of sight of all students on machines. - Prevent cords from becoming trip hazards in high-traffic areas. - Tag and remove defective equipment from service. - Arrange equipment to permit safe, efficient work practices and ease of movement and clean-up. - Make adjustments to the machine when the machine is stopped. - Plug the cord into the appliance first, and then plug the cord into the socket. - Keep food and beverages away from the machine. - Keep hands away from the light or the needle. - Inspect the cords and plugs for fraying and exposed wires. - Check the settings before you start. - Work at a safe speed. - Ensure that correct lighting is provided. - Ensure ergonomic set-up of machine and furniture. - Use the right needle for the chosen fabric and stabilizer weight. - Choose the proper-sized hoop for the embroidery area. - Follow the school safety policy. - Provide and document ongoing safety education and training, including mandatory equipment and tool orientation. - Ensure tools are used under competent supervision. - Ensure repetitive tasks are properly paced. - Post important emergency telephone numbers and contacts. - Post lockdown procedures. - Post fire drill procedures. - Establish handwashing practices. - Ensure fire extinguishers are strategically located and marked. - Provide a first aid kit and supplies, including an eyewash station and biohazard disposal. - Correctly place and maintain smoke detectors and heat sensors. |  |
| Personal Protective Equipment (PPE) |  |
| - safety glasses or eye protection |  |