| CLUSTER | MDC: MEDIA, DESIGN, AND COMMUNICATION ARTS |
| --- | --- |
| OCCUPATION | Fashion |
| HAZARD CONTROLS | EQUIPMENT—IRON AND IRONING BOARD |
| Engineering Controls | iron and ironing board (domestic) adapted from © Africa Studio/44454294/Fotolia and Hemera/Thinkstock |
| - n/a |  |
| Administrative Controls |  |
| - Provide adequate instruction on lab routines, job procedures, and equipment operation. - Keep the work area clean and tidy at all times. - Post safety signage. - Follow the manufacturer’s directions for operation, maintenance, and safe practices. - Inspect the iron and ironing board regularly to ensure safe operation. - Use bottled water in the iron. - Clean up water spills when filling the water reservoir of the iron. - Remove cooled water from the iron before storing the iron. - Ensure a clear line of sight of all students on equipment using heat sources. - Prevent cords from becoming trip hazards in high-traffic areas. - Tag and remove defective equipment from service. - Arrange the equipment and the work area to permit safe, efficient work practices and ease of movement and clean-up. - Make temperature adjustments to the iron before you start. Use the correct temperature setting for the chosen fabric. - Plug the cord into the appliance first, and then plug the cord into the socket. - Keep food and beverages away from the iron and ironing board. - Use a well-balanced, rigid ironing board that has a well-padded and tight-fitting iron cover. - Keep hands away from soleplate and steam vents. - Inspect the cord and plugs for fraying and exposed wires. - Ensure correct lighting is provided. - Ensure ergonomic set-up of the heat source and furniture. - Follow the school safety policy. - Provide and document ongoing safety education and training, including mandatory equipment and tool orientation. - Ensure equipment is used under competent supervision. - Ensure repetitive tasks are properly paced. - Post important emergency telephone numbers and contacts. - Post lockdown procedures. - Post fire drill procedures. - Establish handwashing practices. - Ensure fire extinguishers are strategically located and marked. - Provide a first aid kit and supplies, including an eyewash station and biohazard disposal. - Correctly place and maintain smoke detectors and heat sensors. |  |
| Personal Protective Equipment (PPE) |  |
| - n/a |  |