| MDC: Media, design, and communication arts |  |  |  |
| --- | --- | --- | --- |
| Fashion |  |  |  |
| task | Using an Iron and an Ironing Board |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - electrical - slips, trips, and falls - heat and steam - ironing board may tip over when pulling fabric into position for ironing |  | - electric shock - bruising - burns |
| Chemical | - n/a |  | - n/a |
| Biological | - bacteria/viruses - blood - body fluids - allergy to fabric |  | - Where others have touched the iron, possible contamination via viruses/bacteria may occur. - allergy reaction (e.g., skin reactions) |
| Ergonomic | - repetitive motion - improper work surface height - handling materials and manipulating heavier or larger pieces of fabric when ironing or pressing fabric - poor lighting - poor ventilation - poor posture |  | - possible ergonomic conditions (e.g., carpal tunnel syndrome) - bodily injury - eye strain |
| Psychological | - fatigue - stress |  | - inattentiveness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check for electrical cord damage. - Report any hazards. - Adjust the work surface height appropriately for the user. - Ensure the work environment has excellent ventilation. - Obtain adequate instruction on lab routines, job procedures, and equipment as necessary. - Choose the correct temperature and steam setting on the iron for the fabric to be pressed. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Make adjustments to the temperature and steam settings on the iron prior to starting the task. Use the correct temperature setting for the chosen fabric. - Use correct posture. - Put away all tools, equipment, and materials in their proper storage areas. Allow the iron to cool properly before storing. - Keep the work area clean as you work and make sure to clean up when you are done. Clean up water spills immediately. - Establish handwashing practices. |  | - Keep fingers and hands away from heated parts. - Prevent the cord from becoming a trip hazard. - Unplug the iron when finished. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |