| NAT: NATURAL RESOURCES |  |  |
| --- | --- | --- |
| Forestry |  |  |
| task | Using a Handsaw |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - lifting and handling loads - repetitive motion - hot blade - moving parts; e.g., struck by blade, caught between tree and blade - vibration - embedded objects in trees (e.g., nails) | - musculoskeletal injuries - contusions - lacerations - abrasions - pinching - burns - puncture wounds - embedded objects |
| Chemical | - n/a | - n/a |
| Biological | - bacteria/viruses - blood - body fluids | - illness |
| Ergonomic | - poor or awkward posture - repetitive movement - muscular strength and endurance - vibration | - musculoskeletal injuries |
| Psychological | - fatigue - stress | - inattentiveness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check the stock being cut for nails, knots, and other objects that may damage or buckle the saw.* - Inspect PPE and replace if required. - Report any hazards. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on required PPE. - Secure loose clothing, tie back long hair, and remove jewellery. - Use tools and equipment under competent supervision. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Choose a saw handle that keeps your wrist in a natural position on the horizontal plane.* - Choose a saw with a handle opening that is at least 12 cm (5 in) long and 6 cm (2.5 in) wide and slanted at a 15° angle.* - Start the cut by placing your hand beside the cut mark with your thumb upright and pressing against the blade. Start to cut carefully and slowly to prevent the blade from jumping. Pull upward until the blade bites. Start with a partial cut, and then set the saw at the proper angle.* - Apply pressure on the downstroke only.* - Hold the stock being cut firmly in place. Use a helper, a supporting bench, or vise to support long stock if required.* - Keep the teeth and blades properly set.* - Keep the saw blades clean. Refer to http://www.ccohs.ca/oshanswers/ - safety_haz/hand_tools/handsaw.html. - Protect the teeth of the saw when not in use. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean it up when you are done. - Establish handwashing practices. | - n/a |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |

Content throughout this document that identified with * was Provided by: Hand Tools, Hand Saw. http://www.ccohs.ca/oshanswers/safety_haz/hand_tools/handsaw.html OSH Answers, Canadian Centre of Occupational Health and Safety (CCOHS), 2010. Reproduced/Adapted with the permission of CCOHS, 2013