| NAT: NATURAL RESOURCES |  |  |
| --- | --- | --- |
| Forestry |  |  |
| task | Using a Planting Bag |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - lifting and handling loads - repetitive motion - slips, trips, and falls - moving parts; e.g., fingers caught in buckles or zippers | - musculoskeletal injuries - contusions - lacerations - abrasions - pinching |
| Chemical | - n/a | - n/a |
| Biological | - bacteria/viruses - body fluids - blood - allergens | - illness - asthma or other allergy symptoms |
| Ergonomic | - poor or awkward posture while wearing the bag or putting the bag on - repetitive movement - muscular strength and endurance - product design (fit, weight distribution, and size) | - musculoskeletal injuries |
| Psychological | - fatigue - stress | - inattentiveness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. Inspect the straps of the backpack for damage (e.g., broken stitching or damaged or missing hardware). Ensure the bag is a proper fit and a safe weight to lift. - Inspect required PPE and replace if required. - Report any hazards. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Use tools and equipment under competent supervision. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Place the planting bag on the ground to fill it. - When the bag has been filled, crouch down in the middle of the bag and position the straps over your shoulders. Be sure the load feels balanced before standing up. - From the crouching position, slowly lift through your legs to a standing position. Keep your back straight. Avoid twisting and bending to the side while lifting. - Adjust the waist strap to help carry the load. - Adjust the shoulder straps to keep the load close to your back. - Always fasten the chest strap if there is one. - Keep your work area clean as you work and make sure to clean up when you are done. - Put away all tools, equipment, and materials in their proper storage areas. - Establish handwashing practices. | - Do not lift a load if you are not convinced that you can handle the load safely. - Always bend at the knees when lifting a load. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |