| NAT: NATURAL RESOURCES |  |  |  |
| --- | --- | --- | --- |
| Forestry |  |  |  |
| task | Using Wedges |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - lifting and handling loads - repetitive motion - stuck between wedge and tree or between maul and wedge - vibration from maul |  | - musculoskeletal injuries - contusion - abrasion - pinching |
| Chemical | - n/a |  | - n/a |
| Biological | - bacteria/viruses - blood - body fluids - allergens |  | - illness - asthma or other allergy symptoms |
| Ergonomic | - poor or awkward posture - repetitive movement driving wedge in with maul - muscular strength and endurance - vibration while striking wedges into tree |  | - bodily injury |
| Psychological | - fatigue - stress |  | - inattentiveness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. - Inspect PPE and replace if required. - Report any hazards. - Be sure you have the appropriate size and correct number of wedges. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Use tools and equipment under competent supervision. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Follow all instructions for safe chainsaw use and safe tree falling. - Using a chainsaw, cut one-quarter of the tree trunk’s diameter at a 50°–60° angle. - Create a 90° angle between the top and bottom cut by making a bottom cut at a 30° to 40° angle to meet the top cut. - On the opposite side of the tree, make the felling cut 5 to 7.5 cm (2 to 3 in) above where the front side cuts meet. Leave one-tenth the diameter of the tree to create a hinge. - Insert a wedge into the back cut immediately. Stack wedges to create the necessary lift. - Drive wedges using a maul until the tree begins to fall. - Make sure to move a safe distance away before the tree falls. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. |  | - The larger the tree, the larger the wedge necessary to safely fall the tree; however, a wedge that is too large for a tree can induce a fall in an unwanted direction. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |