| NAT: NATURAL RESOURCES |  |  |  |
| --- | --- | --- | --- |
| Wildlife |  |  |  |
| task | Using a Crossbow |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - lifting and handling loads - repetitive motion - moving parts: struck by string or caught in mechanical release - vibration |  | - death - musculoskeletal injuries - contusions - lacerations - abrasions - pinching - bleeding - puncture wounds |
| Chemical | - n/a |  | - n/a |
| Biological | - allergens: plants and pollen - bacteria/viruses - blood - body fluids |  | - allergy reactions - anaphylactic shock - illness - infection |
| Ergonomic | - poor or awkward posture - repetitive movement - lack of muscular strength to draw bow - bow not properly fitted to user |  | - bodily injury |
| Psychological | - fatigue - stress - fear |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check for cracks, dents, breaks, separating laminates, peeling glass, and defects in mechanical parts. - Check the bowstring regularly, and replace the bowstring if it becomes worn or frayed. - Report any hazards. - First-time bow hunters should be measured for their draw length. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on all required PPE. - Secure loose clothing, and tie back long hair. - Read and follow the manufacturer’s instructions, precautions, and directions. - Follow the school safety policy. - Point the crossbow towards the ground. - Nock the arrow. - Slide the safety into the fire position. - Pull back the trigger to check that the safety is off. - Place one foot through the stirrup located at the front of the crossbow. - Grab the string on opposite sides of the barrel, and pull the string up under the arrow retention spring. - Slowly release the string to be sure it is now caught by the trigger latch. - Point the nose of the crossbow towards the target. - Align the bolt into the flight groove with tip forward. - Slide the bolt under the retention spring to complete the loading procedure. - Place the non-dominant hand under the forestock. - Grip the handle with the dominant hand. - Support the stock of the crossbow against the shoulder. - Keeping the eye at least 6.5 cm (2.5 in) from the scope or rear peep, look down the scope or rear peep to aim. - Squeeze the trigger smoothly using the forefinger to release bolt. - Put away all tools, equipment, and materials in their proper storage areas. - Clean the work area, - Use proper hygiene. Establish handwashing practices. |  | - Only point the bow and bolt in a safe direction. - Only nock a bolt when it’s safe to shoot. - Be sure of your target and what is in front of the target, immediately behind the target, and beyond the target. - Never shoot an arrow over a ridge. - Only shoot when there is a safe range or shooting area and a safe backstop or background. - Avoid dry-firing a bow. - Do not shoot a bolt straight up in the air. - For target practice, use a straight-tip bolt. - Keep the non-trigger hand where it will not be struck by the bolt, string, or limbs when shooting the crossbow. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |