| NAT: NATURAL RESOURCES |  |  |  |
| --- | --- | --- | --- |
| Wildlife |  |  |  |
| task | Wearing a Personal Flotation Device (PFD) |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - moving parts: buckles and zippers |  | - contusions - lacerations - abrasions - pinching |
| Chemical | - n/a |  | - n/a |
| Biological | - n/a |  | - n/a |
| Ergonomic | - poor or awkward posture when taking on or off |  | - bodily injury |
| Psychological | - fatigue - stress - fear |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check for broken buckles or broken zippers and other wear and tear. - Check to see that PFD is proper fit for size and weight of wearer. - Check to see that PFD is approved by Transport Canada or the Canadian Coast Guard. - Report any hazards and replace ripped or damaged PFD. - Make sure that a working whistle is attached to the PFD. - At the beginning of each season, test the buoyancy of the PFD by performing the following procedure: - Put on and properly fit the PFD or life jacket. - Wade into chest-deep water. - Bend your knees and float onto your back. - Ensure the PFD or life jacket keeps your chin above the water and permits proper breathing. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Read and follow the manufacturer’s instructions, precautions, and directions. - Follow the school safety policy. - Although a student should never enter a boat without wearing a PFD, it is valuable to have students be as familiar with PFDs as possible. Therefore, have students practise putting on a life jacket in water in a supervised area prior to any boating activities. - To practise putting on a life jacket in the water, follow these steps: - Spread the flotation device open with the inside facing up and out of the water and the neck facing towards you. - Extend your arms through the arm openings. - Lift your arms above your head. - Lie backwards and pull the flotation device around your upper body. - Fasten the zipper, straps, buckles, and/or ties to ensure a snug fit. - Put away all tools, equipment, and materials in their proper storage areas. - Clean the work area. - Establish handwashing practices. |  | - Approved status of PFDs is void if the flotation device has been damaged, altered, or repaired or if the label or stamp indicating approval is illegible. - Never use a PFD as a cushion, bumper, or fender. - Never dry clean or use harsh chemicals to clean a PFD. To clean a PFD, wash with warm soapy water and rinse well. - The PDF should be air dried but not in direct sunlight and away from heat sources. - PFDs will not keep an unconscious person face up out of water. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |