| NAT: NATURAL RESOURCES |  |  |  |
| --- | --- | --- | --- |
| Wildlife |  |  |  |
| task | Using a Handheld Ice Auger |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - lifting and handling loads - repetitive motion - slips, trips, and falls - moving parts - vibration - sharp blade |  | - musculoskeletal injuries - contusions - lacerations - abrasions - pinching - puncture wounds |
| Chemical | - cleaning agent, lubricant oil |  | - chemical burns - skin irritation - lung damage - headache - dizziness - loss of consciousness |
| Biological | - bacteria/viruses - blood - body fluids |  | - illness |
| Ergonomic | - poor or awkward posture - repetitive motion - lack of muscular strength, endurance, flexibility, and/or balance - vibration - incorrect handle height for user |  | - bodily injury |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. - Inspect PPE and replace if required. - Report any hazards. - Read, understand, and follow the manufacturer's operating manual and safety decals on the equipment before operating the equipment. (Go to http://www.ccohs.ca for more information.) - Inspect and tighten blade screws and all joints. - Ensure that the blade is sharp. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on all required PPE. Wear non-slip footwear. - Secure loose clothing, and tie back long hair. Remove jewellery. - Read and follow the manufacturer’s instructions, precautions, and directions. - Follow the school safety policy. - Set the handle to correct height for the user—that is, no higher than the user’s underarm when the blade is touching the ice. - Uncover the blade. - Rest the blade on the ice. - Press down on the top knob of the handle with one arm close to the right shoulder if right-handed. - While pressing down on the handle knob, use the other hand to turn the ice auger clockwise. - Apply steady downward pressure. - Once through the ice, turn the auger counterclockwise while pulling up on the handle. - Cover the blade. - Put away all tools, equipment, and materials in their proper storage areas. - Clean the work area. - Establish handwashing practices. |  | - Always cover blade when not in use. - Keep the blade sharpened. - Secure the auger for transport. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |