| CLUSTER | TMT: TRADES, MANUFACTURING, AND TRANSPORTATION |
| --- | --- |
| OCCUPATION | Construction |
| HAZARD CONTROLS | CONSUMABLES—FINISHES (e.g., oil, varnish, lacquer, paste wax, polyurethane, paint) |
| Engineering Controls | Adapted from iStockphoto/Thinkstock |
| - fume extraction - explosion-proof finishing room - fireproof rag disposal bin (if a pickup service is used) - Refer to the Material Safety Data Sheet for the required level of ventilation. |  |
| Administrative Controls |  |
| - Use less toxic, water-based, zero volatile organic compound (VOC) alternative finishes. - Wipe the respirator with a respirator wipe before and after use (if shared). - Follow rag disposal protocol. - Provide initial training to include a thorough review of hazards and accidents associated with the job. - Follow the school safety policy. - Provide and document ongoing safety education and training, including mandatory equipment and tool orientation. - Outline the “imminent danger/right to refuse” procedures. - Ensure hazard signs and tags are in place. - Tag and remove defective tools from service. - Ensure tools are used under competent supervision. - Ensure the tool is appropriate for the work to be done. - Schedule work to avoid overcrowding. - Ensure repetitive tasks are properly paced. - Post important emergency telephone numbers and contacts. - Post lockdown procedures. - Post fire drill procedures. - Establish handwashing practices. - Locate the emergency electrical shut-off. - Ensure fire extinguishers are strategically located and marked. - Provide a first aid kit and supplies, including an eyewash station and biohazard disposal. - Ensure that the master control panel and panel breakers are properly labelled. - Keep manufacturers’ manuals for the finishes. - Provide an ergonomically appropriate work area. - Ensure that correct lighting is available. - Correctly place and maintain smoke detectors and heat sensors. - Post relevant safety and health facts in the work area. - Ensure that students wear safe attire: no loose clothing or jewellery should be worn. - Ensure that long hair is tied back and/or secured under a hat. - Ensure WHMIS training is available. - Provide up-to-date MSDS. - Follow lock-out procedures for performing maintenance. |  |

| Personal Protective Equipment (PPE) |  |
| --- | --- |
| - chemical splash goggles - closed footwear - impervious gloves - apron - organic vapour respirator, if required by the Material Safety Data Sheet - Refer to the Material Safety Data Sheet for the specific finish being used. |  |