| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |  |
| --- | --- | --- | --- |
| Construction |  |  |  |
| task | Using Abrasives |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - repetitive motion - contact with material |  | - repetitive motion injury - abrasions |
| Chemical | - wood dust - poisons/toxins, including fibreglass, asbestos, and lead |  | - respiratory damage |
| Biological | - n/a |  | - n/a |
| Ergonomic | - n/a |  | - n/a |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect the tools, equipment, and materials for damage and other hazards. - Inspect PPE and replace if required. - Report any hazards. - Have unknown materials or surfaces lab tested for lead before sanding. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Position the material on the downdraft sanding table. - Install abrasive on the chosen block or tool. - Sand in an even and methodical pattern. - Ensure all wood dust is contained at the work station and cleaned up when complete. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. |  | - Do not breathe in vapours or dust. - Do not sand unknown surfaces or materials, as old paint may contain lead. - Use the correct size of abrasive for the tool. Paper discs should not protrude past the edges of the backing pad. - Use the appropriate abrasive grit for the procedure. - Refer to the MSDS for wood dust. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |