| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |
| --- | --- | --- |
| Construction |  |  |
| task | Using a Band Saw |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - flying objects - electrical shock - lifting and handling loads - noise - repetitive motion - moving parts - sharp blade | - bruising - pinching - cuts/amputations - hearing loss - eye irritation and/or eye damage - repetitive motion injury |
| Chemical | - wood dust - human-made material particulates (e.g., lexan, polycarbonate, laminates, MDF, and plywood) | - respiratory damage |
| Biological | - bacteria/virus - blood - body fluids | - illness |
| Ergonomic | - lifting heavy stock materials - vibration - lighting | - bodily injury - eye strain |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect the tools, equipment, and materials for damage and other hazards. For example, check for electrical cord and switch damage. - Inspect the material for knots, splits, warp, nails, staples, or other defects. - Inspect PPE and replace if required. - Report any hazards. - Check that all guards are in place and working correctly. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Position the material on the machine table. - Adjust the blade guard/guide to ¼ in above the material. - Turn on the machine. - Feed the material into the blade and complete the cut, maintaining a 5-cm or 2-in margin of safety. Use a push stick if necessary. Feed the stock at a steady controlled pace, no faster than the blade can easily cut. - Hold the stock flat on the table. - Turn the machine off, and wait for the blade to completely stop. - Remove the material from the band saw table. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. | - Do not breathe in vapours or dust. - Use a stable and comfortable stance. - Never put your fingers in line with the blade. - Focus your attention on the point of the cut (where the blade enters the wood). - If you hear a clicking noise (indicating a broken blade), turn the machine off and move your hands away from blade until the machine stops. - Use relief cuts for cutting curves. - Turn the machine off before backing out of a cut. - Use a V-block for cutting round stock, such as dowels. - Feed the stock at a steady controlled pace, no faster than the blade can easily cut. - Hold the stock flat on the table |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |