| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |  |
| --- | --- | --- | --- |
| Construction |  |  |  |
| task | Using a Bench Grinder or Bench-Top Grinder |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - electrical shock - eye hazards: chipping, grinding, flying metal or dirt - lifting and handling loads - excessive noise - repetitive motion - slips, trips, and falls - hot parts - moving parts - grinder wheel disintegration |  | - death - bruising - pinching - cuts/amputations - hearing impairment - eye irritation and/or eye injury - burns - chronic pain - cancer |
| Chemical | - fumes: aluminum, beryllium, cadmium oxides, chromium, copper, fluorides, iron oxide, lead, manganese, molybdenum, nickel, vanadium, zinc oxides - gases: carbon monoxide, hydrogen fluoride, nitrogen oxide, ozone, phosphine - organic vapours: aldehydes (such as formaldehyde), di-isocyanates, phosgene |  | - Acute effects (occurring immediately or quickly) include metal fume fever; damage to the respiratory tract; sore and dry throat; chest pain; breathing difficulty; skin irritation; irritation of the eyes, nose, and throat; nausea; dermatitis; headache; dizziness; and muscular weakness. - Chronic effects (appearing over time) include kidney damage; emphysema; cancer; bone and joint problems; excess fluid in the lungs; siderosis; chronic effects to the nervous system, kidneys, digestive system, and mental capacity; lead poisoning; bronchitis; retinitis; pneumonia; and asphyxiation. - oxygen deficiency - death |
| Biological | - allergens: fumes and gases - bacteria/virus - blood - body fluids |  | - asthma and other allergy symptoms - illness |
| Ergonomic | - lifting, repetitive motion - work surface height - lighting |  | - bodily injury - eye strain |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, inspect the material for defects. Check for electrical cord and switch damage. Check for grinding wheel cracks or damage. - Inspect PPE and replace if required. - Report any hazards. - Check that all guards are in place and working correctly. - Adjust the work surface height appropriately for the user. - Ensure the bed is as close as possible to the grinding wheel. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Always position the material on the tool rest or grinding jig. Never do freehand grinding. - Adjust the tool rest or grinding jig. - Hold the stock flat on the table. - Turn on the machine. - Grind the material to the desired dimensions. Grind on the upper exposed quarter of the wheel only. - Pause frequently to allow cooling. - Use a wheel dresser to dress the grinding stone as it loses its shape or fills with debris. - Turn the machine off and wait for the wheel to completely stop. - Remove the material from the tool rest. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. |  | - Do not breathe in dust. - Use a stable and comfortable stance. - Do not grind aluminum. - Specialized procedures should be performed by the teacher or by the student only after satisfactory training and with teacher supervision. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |