| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |
| --- | --- | --- |
| Construction |  |  |
| task | Using a Compound Mitre Saw |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - flying objects - electrical shock - lifting and handling loads - noise - repetitive motion - moving parts - sharp blade - vibration | - death - bruising - pinching - cuts/amputations - hearing impairment - eye irritation and/or eye injury - repetitive motion injury |
| Chemical | - wood dust - human-made material particulates (e.g., lexan, polycarbonate, laminates, MDF, and plywood) | - respiratory damage |
| Biological | - bacteria/virus - blood - body fluids | - illness |
| Ergonomic | - heavy stock materials - lighting | - bodily injury - eye strain |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect the tools, equipment, and materials for damage and other hazards. For example, check for electrical cord and switch damage. - Inspect the material for knots, splits, warp, nails, staples, or other defects. - Inspect PPE and replace if required. - Report any hazards. - Check that all guards are in place and working correctly. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Adjust the saw to the desired angles, and lock it in place. - Position and hold the material on the machine table and against the fence, maintaining a 10-cm or 4-in margin of safety. Ensure there is no gap between the wood and the fence (joint edge of stock if necessary). - Clamp small pieces in place. - Turn the machine on. - Feed the blade into the material, and complete the cut. Make sure to feed the blade at a steady, controlled pace that is no faster than the blade can easily cut. - Hold the stock flat on the table. - Turn the machine off, and wait for the blade to completely stop. - Raise the saw/blade. - Remove the material from the saw table. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. | - Do not breathe in vapours or dust. - Use a stable and comfortable stance. - Focus your attention on the area in which the material will be cut first. - When cutting small pieces, use a sacrificial fence for additional support and let the blade come to a complete stop in the down position. - If using a stop block for repetitive cuts, let the blade come to a complete stop in the down position. - Feed the blade at a steady, controlled pace, no faster than the blade can easily cut. - Hold the stock flat on the table. - Disconnect the saw from the power source when changing blades. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |