| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |  |
| --- | --- | --- | --- |
| Construction |  |  |  |
| task | Using Edged Hand Tools (planes, chisels, hand saws) |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - repetitive motion - sharp blade |  | - chronic pain - cuts and lacerations - pinching |
| Chemical | - wood dust |  | - respiratory illness |
| Biological | - bacteria/virus - blood - body fluids |  | - illness |
| Ergonomic | - workbench height - lighting |  | - bodily injury - eye strain |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect the tools, equipment, and materials for damage and other hazards. - Inspect PPE and replace if required. - Report any hazards. - Inspect the condition of the tool’s cutting edge for sharpness and defects. - Check the adjustment of the tool, if applicable. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Clamp the work piece to the work surface or a vice so that the work piece doesn’t move around. - Hold the tool with both hands. - Push the tool away from your body and away from others. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. |  | - Do not breathe in dust. - Never check blade sharpness with your skin. Use paper or perform a skid test on the barrel of a plastic pen. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |