| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |
| --- | --- | --- |
| Construction |  |  |
| task | Using a Pneumatic Nailer or Nail Gun |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - flying objects - electrical shock - noise - lifting and handling loads - repetitive motion - moving parts - vibration | - bruising - pinching - cuts/piercing - hearing impairment - eye irritation and/or eye damage - chronic pain |
| Chemical | - n/a | - n/a |
| Biological | - bacteria/virus - blood - body fluids | - illness |
| Ergonomic | - lifting heavy stock materials - lighting | - bodily injury - eye strain |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check for electrical cord and switch damage. - Inspect the material for knots, splits, warp, nails, staples, or other defects. - Inspect PPE and replace if required. - Report any hazards. - Check that all guards are in place and working correctly. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Position and hold or clamp the material in a way that you cannot be hit by a deflected nail. This is especially important when nailing along an edge. - Adjust the pressure for the material type and thickness. - Hold the tool firmly. - Push the safety tip against the material. - Pull the trigger to fire the nail or brad. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. | - Use a stable and comfortable stance. - Use an air regulator to ensure the manufacturer’s recommended air pressure is used. - Never point the nailer at yourself or others. - Do not carry the tool with the trigger pulled. Bumping the safety tip will result in the discharge of a nail. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |