| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |
| --- | --- | --- |
| Construction |  |  |
| task | Using Portable Power Tools |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - flying objects - electrical shock - lifting and handling loads - repetitive motion - moving parts - vibration | - bruising - pinching - cuts/amputations - eye irritation and/or eye injury - chronic pain |
| Chemical | - wood dust - human-made material particulates (e.g., lexan, polycarbonate, laminates, MDF, and plywood) | - respiratory damage |
| Biological | - bacteria/virus - blood - body fluids | - illness |
| Ergonomic | - lifting heavy stock materials - lighting | - bodily injury - eye strain |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check for electrical cord and switch damage. - Inspect the material for knots, splits, warp, nails, staples, or other defects. - Inspect PPE and replace if required. - Report any hazards. - Check that all guards are in place and working correctly. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Position and clamp the material. - Adjust the speeds and angles for the specific operation. - Hold the tool firmly in two hands. - Pull the power trigger. - Feed the bit or blade into the material and complete the action. - Let go of the power trigger to turn the tool off, and wait for the bit or blade to completely stop before putting the tool down. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. | - Do not breathe in vapours or dust. - Use a stable and comfortable stance. - Use two hands on the tool. - Clamp or secure the material before cutting. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |