| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |
| --- | --- | --- |
| Construction |  |  |
| task | Using a Portable Router |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - flying objects - electrical shock - lifting and handling loads - noise - repetitive motion - moving parts - vibration | - bruising - pinching - cuts/amputations - hearing impairment - eye irritation and/or eye damage - chronic pain |
| Chemical | - wood dust - human-made material particulates (e.g., MDF and plywood) | - respiratory damage |
| Biological | - bacteria/virus - blood - body fluids | - illness |
| Ergonomic | - lifting heavy stock materials - lighting | - bodily injury - eye strain |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check for electrical cord and switch damage. - Inspect the material for knots, splits, warp, nails, staples, or other defects. - Inspect PPE and replace if required. - Report any hazards. - Check that all guards are in place and working correctly. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Position and clamp the material on the work table. - Adjust the guide fence and template, or use an edge bit with a guide bearing. - Install the appropriate router bit (with the router unplugged). - Adjust the router base for the desired depth of cut. - Always use two hands on the router, and hold the router firmly with its base on the material and its bit clear of the cut. - Use successive passes with increasing bit depth, when possible, until the final depth is achieved. - Turn the router on. - Feed the bit into the material against the direction of bit rotation and complete the cut. Cut against the direction of bit rotation. - Turn the tool off and wait for the bit to completely stop before removing it from the material and putting the tool down. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. | - Do not breathe in vapours or dust. - Use a stable and comfortable stance. - Install the router bit with ¾ of its shank buried in the collet, but do not bottom out the bit before tightening. - Unplug the router when changing bits, when making adjustments, or when the router is not in use. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |