| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |
| --- | --- | --- |
| Construction |  |  |
| task | Using a Scroll Saw |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - flying objects - electrical - lifting and handling loads - noise - repetitive motion - moving parts - sharp blade - vibration | - bruising - pinching - cuts/amputations - hearing impairment - eye irritation and/or eye damage - electrical shock - chronic pain |
| Chemical | - wood dust - human-made material particulates (e.g., lexan, polycarbonate, laminates, MDF, and plywood) | - respiratory damage |
| Biological | - n/a | - n/a |
| Ergonomic | - lifting heavy stock materials - lighting | - bodily injury - eye strain |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check for electrical cord and switch damage. - Inspect the material for knots, splits, warp, nails, staples, or other defects. - Inspect PPE and replace if required. - Report any hazards. - Check that all guards are in place and working correctly. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Position the material on the machine table. - Adjust the material hold-down, blade tension, and blade speed for the specific material and operation. - Turn the machine on. - Feed the material into the blade and complete the cut, maintaining a 5-cm or 2-in margin of safety. Use a push stick if necessary. Feed the stock at a steady, controlled pace, no faster than the blade can easily cut. - Hold the stock flat on the table. - Turn the machine off, and wait for the blade to completely stop. - Remove the material from the scroll saw table. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. | - Do not breathe in vapours or dust. - Use a stable and comfortable stance. - Never put your fingers in line with the cut. - Focus your attention on the point of cut. - Use relief cuts for cutting tight curves. - Turn the machine off before backing out of a cut. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |