| TMT: trades, manufacturing, & transportation |  |  |  |
| --- | --- | --- | --- |
| Electro-Technologies |  |  |  |
| task | Using a Bench Grinder |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - electrical shock - eye hazards: chipping, grinding, flying metal or dirt - lifting and handling loads - excessive noise - repetitive motion - slips, trips, and falls - hot parts - moving parts |  | - bruising - pinching - cuts/amputations - hearing loss - eye injury - repetitive motion injury |
| Chemical | - fumes: aluminum, beryllium, cadmium oxides, chromium, copper, fluorides, iron oxide, lead, manganese, molybdenum, nickel, vanadium, zinc oxides - gases: carbon monoxide, hydrogen fluoride, nitrogen oxide, ozone, phosphine - organic vapours: aldehydes (such as formaldehyde), di-isocyanates, phosgene |  | - Acute effects (occurring immediately or quickly) include metal fume fever; damage to the respiratory tract; sore and dry throat; chest pain; breathing difficulty; skin irritation; irritation of the eyes, nose, and throat; nausea; dermatitis; headache; dizziness; and muscular weakness. - Chronic effects (appearing over time) include kidney damage; emphysema; cancer; bone and joint problems; excess fluid in the lungs; siderosis; chronic effects to the nervous system, kidneys, digestive system, and mental capacity; lead poisoning; bronchitis; retinitis; pneumonia; and asphyxiation. - oxygen deficiency - death |
| Biological | - allergens: fumes and gases - bacteria/viruses - blood - body fluids |  | - asthma and other allergy symptoms - illness |
| Ergonomic | - lifting, repetitive motion - work surface height - lighting |  | - bodily injury - eye strain |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, inspect the material for defects. Check for electrical cord and switch damage. - Inspect PPE and replace if required. - Report any hazards. - Check that all guards are in place and working correctly. - Adjust the work surface height appropriately for the user. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on all required PPE. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Always position the material on the tool rest or grinding jig. Never do freehand grinding. - Adjust the tool rest or grinding jig. - Hold the stock flat on the table. - Turn on the machine. - Grind the material to the desired dimensions. Grind on the upper exposed quarter of the wheel only. - Pause frequently to allow cooling. - Use a wheel dresser to dress the grinding stone as it loses its shape or fills with debris. - Turn the machine off and wait for the wheel to completely stop. - Remove the material from the tool rest. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. |  | - Do not breathe in dust. - Use a stable and comfortable stance. - Do not grind aluminum. - Specialized procedures should be performed by the teacher or by the student only after satisfactory training and teacher supervision. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |