| Task hazard analysis |  |  |  |
| --- | --- | --- | --- |
| Electro-Technologies |  |  |  |
| task | Using a Digital Multimeter (DMM) |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - electrical |  | - electrical shock - burns |
| Chemical | - n/a |  | - n/a |
| Biological | - bacteria/viruses - blood - body fluids |  | - Where others have touched devices, possible contamination via viruses/bacteria may occur. |
| Ergonomic | - n/a |  | - n/a |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, inspect meter leads. - Report any hazards. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| If DMM is to be used as ohmmeter: - Place leads in correct jacks: red is positive and black is negative. - Ensure the component to be tested is de-energized. - Connect the ohmmeter across (in parallel) with the component to be tested. - Unless the ohmmeter is auto-ranging, select the range that will give the most accurate reading. - Observe the digital read-out. - Disconnect the ohmmeter, and switch off. If the DMM is to be used as a voltmeter: - Place leads in correct voltmeter jacks: red is positive and black is negative. There is no polarity when measuring AC, but polarity must be observed when measuring DC. - Unless voltmeter is auto-ranging, the ranges are finite. Start on the highest range and work down. - The voltmeter is connected in parallel (with the load). - Observe the digital read-out. - Disconnect the voltmeter. For all uses: - Use tools and equipment under competent supervision. - Secure loose clothing, and tie back long hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Put away all tools, equipment, and materials in their proper storage areas. - Clean the work area. - Establish handwashing practices. |  | If DMM is to be used as ohmmeter: - Make sure the component to be tested is de-energized before attempting to connect the ohmmeter. If DMM is to be used as voltmeter: - The maximum rating of the voltmeter for any range must not be exceeded. - Have the work inspected by the instructor before energizing for the first time. - Use caution when connecting probes to circuitry. Always treat all circuitry as if it is alive, and keep fingers clear. If DMM is to be used as ammeter/milliammeter: - Make sure the circuit to be tested is de-energized before attempting to open the circuit to make connections. - Make sure the milliammeter/ammeter is connected in series with the load. This means the circuit under test must be opened (taken apart). - The maximum rating of the milliammeter/ammeter for any range must not be exceeded. - Have the work inspected by the instructor before energizing. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |