| Task hazard analysis |  |  |
| --- | --- | --- |
| Electro-Technologies |  |  |
| task | Using a Drill Press |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - electrical shock - eye hazards: flying objects - lifting and handling loads - repetitive motion - hot parts - moving parts | - death - bruising - pinching - eye irritation and/or eye damage - burns - cuts - punctures - chronic pain - repetitive motion injury |
| Chemical | - wood dust - human-made material particulates (e.g., lexan, polycarbonate, laminates, MDF, and plywood) | - respiratory damage |
| Biological | - bacteria/viruses - blood - body fluids | - Where others have touched devices, possible contamination via viruses/bacteria may occur. |
| Ergonomic | - work surface height - lifting heavy stock materials - repetitive work - lighting | - bodily injury - chronic pain - eye strain |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect the tools, equipment, and materials for damage and other hazards. For example, check for electrical cord and switch damage. - Inspect the material for knots, splits, warp, nails, staples, or other defects. - Inspect PPE and replace if required. - Report any hazards. - Check that all guards are in place and working correctly. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on all required PPE. - Secure loose clothing, and tie back long hair. Loose clothing and long hair has potential to wind around the spindle, jamming between material and the tool rest. - Refer to the manufacturer’s speed recommendations for the material to be drilled (when using an adjustable-speed drill press), and adjust as necessary. - Position the material on the machine table and clamp it flat to the table. Use a sacrificial board when through-drilling. - Adjust the table height. - Turn the machine on. - Plunge the bit slowly and steadily into the material and complete the drilling, maintaining a 5-cm or 2-in margin of safety. Feed the bit at a steady, controlled pace, no faster than the bit can easily cut. - Use clamps for small parts, especially when drilling through the material. Also, use a V-block and clamp for drilling round stock, such as dowels. - Hold the stock flat on the table, with a sacrificial board under the material. - Raise the bit occasionally to clear chips from the bit and the hole, especially when using brad point and foster bits. - Turn the machine off, and wait for the bit to completely stop. - Remove the material from the drill press table. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. | - Do not breathe in vapours or dust. - Use a stable and comfortable stance. - Never leave the chuck key in the drill chuck. - Do not use drill bits with a screw tip. - Use twist bits when drilling plastics. - Feed the bit at a steady, controlled pace, no faster than the bit can easily cut. - Hold the stock flat on the table, with a sacrificial board under the material. - Specialized procedures, such as using a fly-cutter, should be performed by the teacher or by the student only after satisfactory training and teacher supervision. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |