| Task hazard analysis |  |  |  |
| --- | --- | --- | --- |
| Electro-Technologies |  |  |  |
| task | Using Various Meters |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - electrical |  | - electrical shock - burns |
| Chemical | - n/a |  | - n/a |
| Biological | - bacteria/viruses - blood - body fluids |  | - Where others have touched devices, possible contamination via viruses/bacteria may occur. |
| Ergonomic | - n/a |  | - n/a |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, inspect meter leads. - Report any hazards. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Use tools and equipment under competent supervision. - Follow the school safety policy. - Place the leads in the correct jacks. - De-energize the circuit to be tested. - Follow the manufacturer’s instructions. Connect the meter according to the manufacturer’s instructions. - Unless meter is auto-ranging, the ranges are finite. Generally, start on the highest range and work down. Make sure the maximum rating of the meter is not exceeded. - Have the work inspected by the instructor before energizing. - Energize the circuit and observe the digital read-out or the reading from the appropriate scale if the meter is analog. - De-energize the circuit and disconnect the meter. - Restore the circuit under “test.” - Put away all tools, equipment, and materials in their proper storage areas. - Clean the work area. - Establish handwashing practices. |  | - Make sure the circuit to be tested is de-energized before attempting to open the circuit to make connections. - Make sure the maximum rating of the meter is not exceeded. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |