| Task hazard analysis |  |  |
| --- | --- | --- |
| Electro-Technologies |  |  |
| task | Using a Wattmeter |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - electrical | - electrical shock - burns |
| Chemical | - n/a | - n/a |
| Biological | - bacteria/viruses - blood - body fluids | - Where others have touched devices, possible contamination via viruses/bacteria may occur. |
| Ergonomic | - n/a | - n/a |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, inspect meter leads. - Report any hazards. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Use tools and equipment under competent supervision. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Place the leads in the correct jacks for milliammeter/ammeter section and the voltmeter section. There is no polarity. - De-energize the circuit to be tested. - Connect the milliammeter/ammeter in series with the load. - Unless the milliammeter/ammeter section is auto-ranging, the ranges are finite. Stay within the indicated current range. The maximum rating of the milliammeter/ammeter section for any range must not be exceeded. - Connect the voltmeter section in parallel with the load. - Unless the voltmeter section is auto-ranging, the ranges are finite. Stay within the indicated voltage range. The maximum rating of the voltmeter section for any range must not be exceeded. - Have the work inspected by the instructor before energizing. - Energize the circuit and observe the digital read-out or the reading from the appropriate scale if the wattmeter is analog. - If the wattmeter reads backward, reverse the voltmeter section leads. - De-energize the circuit and disconnect the wattmeter. - Restore the circuit under test. - Put away all tools, equipment, and materials in their proper storage areas. - Clean the work area. - Establish handwashing practices. | - Make sure the circuit to be tested is de-energized before attempting to open the circuit to make connections. - Make sure the milliammeter/ammeter section is connected in series with the load. This means the circuit under test must be opened (taken apart). - The maximum rating of the milliammeter/ammeter section for any range must not be exceeded. - Make sure the voltmeter section is connected in parallel with the load. - The maximum rating of the voltmeter section for any range must not be exceeded. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |