| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |  |
| --- | --- | --- | --- |
| Fabrication |  |  |  |
| task | Using a Pneumatic Die Grinder |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - eye hazards: chipping, grinding, flying metal or dirt - fire and explosion - lifting and handling loads - excessive noise - compressed air system - repetitive motion - slips, trips, and falls - hot parts - moving parts |  | - death - bruising - pinching - eye irritation and/or eye damage - hearing impairment - burns - cuts - chronic pain - repetitive motion injuries - cancer |
| Chemical | - fumes: aluminum, beryllium, cadmium oxides, chromium, copper, fluorides, iron oxide, lead, manganese, molybdenum, nickel, vanadium, zinc oxides - gases: carbon monoxide, hydrogen fluoride, nitrogen oxide, ozone, phosphine - organic vapours: aldehydes (such as formaldehyde), di-isocyanates, phosgene - compressed-gas cylinders: argon, carbon dioxide, acetylene, oxygen, MAPP, propane |  | - Acute effects (occurring immediately or quickly) include metal fume fever; damage to the respiratory tract; sore and dry throat; chest pain; breathing difficulty; skin irritation; irritation of the eyes, nose, and throat; nausea; dermatitis; headache; dizziness; and muscular weakness. - Chronic effects (appearing over time) include kidney damage; emphysema; cancer; bone and joint problems; excess fluid in the lungs; siderosis; chronic effects to the nervous system, kidneys, digestive system, and mental capacity; lead poisoning; bronchitis; retinitis; pneumonia; and asphyxiation. - oxygen deficiency - burns - death |
| Biological | - allergens: fumes and gases - bacteria/viruses - blood - body fluids |  | - asthma or other allergy symptoms - illness |
| Ergonomic | - work surface height - lifting - repetitive work |  | - bodily injury - chronic pain |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. - Inspect PPE and replace if required. - Report any hazards. - Adjust the work surface height appropriately for the user. - Check that the ventilation system is operational before starting to grind. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on required PPE. All grinding processes produce hazardous fumes. Minimize your exposure. - Secure loose clothing, and tie back long hair. - Use tools and equipment under competent supervision. - Work in isolation or behind a barrier to minimize exposure of others to harmful flying objects. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. |  | - n/a |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |