| TMT: TRADES, MANUFACTURING, AND TRANSPORTATION |  |  |
| --- | --- | --- |
| Mechanics |  |  |
| task | Using an Abrasive Cut-off Saw (Chop Saw) |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - electrical shock - eye hazards: chipping, grinding, flying metal or dirt - fire and explosion - lifting and handling loads - excessive noise - repetitive motion - slips, trips, and falls from height - hot materials and equipment - moving parts | - death - bruising - pinching - eye irritation and/or eye damage - hearing impairment - burns - chronic pain - cancer - cuts |
| Chemical | - fumes: aluminum, beryllium, cadmium oxides, chromium, copper, fluorides, iron oxide, lead, manganese, molybdenum, nickel, vanadium, zinc oxides - gases: carbon monoxide, hydrogen fluoride, nitrogen oxide, oxygen deficiency, ozone - organic vapours: aldehydes (such as formaldehyde), di-isocyanates, phosgene, phosphine | - Acute effects (appear quickly) include metal fume fever; damage to respiratory tract; sore and dry throat; chest pain; breathing difficulty; skin irritation; irritation of the eyes, nose, and throat; nausea; dermatitis; headache; dizziness; and muscular weakness. - Chronic effects (appear over time) include kidney damage; emphysema; cancer; bone and joint problems; excess fluid in the lungs; siderosis; chronic effects to the nervous system, kidneys, digestive system, and mental capacity; lead poisoning; bronchitis; retinitis; pneumonia; and asphyxiation. |
| Biological | - allergens: fumes and gases - bacteria/viruses - blood - body fluids | - asthma or other allergy symptoms - illness |
| Ergonomic | - work surface height - lifting - repetitive work | - bodily injury - chronic pain |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |

| Pre-operational Safety Checks |  |
| --- | --- |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. - Inspect PPE and replace if required. - Report any hazards. - Adjust the work surface height appropriately for user. |  |
| Safe Work Procedure | Cautions |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Use tools and equipment under competent supervision. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Put away all tools, equipment, and materials in proper their storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. | - n/a |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |