| TMT: TRADES, MANUFACTURING, AND TRANSPORTATION |  |  |  |
| --- | --- | --- | --- |
| Mechanics |  |  |  |
| task | Using an Air Blow Gun |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - eye hazards: flying debris, fluid, or dirt - excessive noise - compressed air system - repetitive motion - slips, trips, and falls - moving parts |  | - eye irritation and/or eye damage - hearing impairment - chronic pain - bruising |
| Chemical | - exposure to solvents |  | - chemical burns - skin irritation - eye injury - particle inhalation |
| Biological | - allergens: fumes and gases - bacteria/viruses - blood - body fluids |  | - asthma or other allergy symptoms - illness |
| Ergonomic | - lifting - repetitive work |  | - bodily injury - chronic pain |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. - Inspect PPE and replace if required. - Report any hazards. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on required PPE. Always wear eye protection when using air tools! Automotive processes produce hazardous fumes and gases. Minimize your exposure. - Secure loose clothing, and tie back long hair. - Use tools and equipment under competent supervision. Automotive repair requires skill and attention; therefore, you must be present and alert! - Follow the manufacturer’s instructions. - Follow the school safety policy. - Put away all tools, equipment, and materials in proper their storage areas. - Do not use the air blowgun to clean brake dust from brake components; the air blowgun will disperse the dust through the workshop. - Do not use a high-pressure air blowgun to disperse liquid solvents or fuels. A low-pressure blowing action can help these volatile materials evaporate more quickly, but a high-pressure air jet could atomize the liquid, which forms a flammable mixture. - Make sure to direct the exhaust of air tools away from yourself and others working in the area. - Do not point the air blowgun at people. - Never use the air blowgun to blow air over yourself or other people. - Make sure that you understand and observe all legislative and personal safety procedures when carrying out tasks with blow guns. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. |  | - n/a |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |