| TMT: TRADES, MANUFACTURING, AND TRANSPORTATION |  |  |  |
| --- | --- | --- | --- |
| Mechanics |  |  |  |
| task | Using a Crimper |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - eye hazards: chipping, flying metal or dirt - lifting and handling loads - repetitive motion - slips, trips, and falls |  | - death - bruising - pinching - eye irritation and/or eye damage - crushing injuries - chronic pain - repetitive motion injuries |
| Chemical | - n/a |  | - n/a |
| Biological | - bacteria/viruses - blood - body fluids |  | - illness |
| Ergonomic | - work surface height - manual handling of steel - lifting, repetitive work |  | - bodily injury - chronic pain |
| Psychological | - fatigue - stress |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. - Inspect PPE and replace if required. - Report any hazards. - Adjust the work surface height appropriately for the user. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on required PPE. - Secure loose clothing, and tie back long hair. - Use tools and equipment under competent supervision. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. |  | - Be aware of and avoid pinch points. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |