# CTS HRH—COS # OCCUPATIONAL HAZARD ASSESSMENT AND CONTROL

Note 1: Alberta’s Occupational Health and Safety Code (OH&S Code) provides specific compliance requirements. This document is meant as a summary or checklists to assist staff comply with the legislation or applicable school jurisdiction administrative policies and regulations.

Note 2: The employee is required to conduct a pre-job hazard assessment before commencing work.

Note 3: This is a general worksite hazard assessment. Depending on the school/instructional area/specific courses taught, some incidents/hazards may or may not be present.

| RISK ANALYSIS (A x B x C = DEGREE OF RISK) |  |  |  |  |
| --- | --- | --- | --- | --- |
| A Frequency of Exposure to Hazard | B Incident/Hazard Probability | C Potential Consequence | Degree of Risk | Risk Classification |
| 1 = Less than once a month 2 = At least once a month 3 = At least once a week 4 = One or more times daily | 1 = Not likely 2 = Remote: not likely by once every 5–20 yrs 3 = Occasional: likely to happen once every 1–5 yrs 4 = Probable: expected to happen one or more times/year | 1 = Negligible (no injury, first aid) 2 = Marginal (medical aid, minor injury or illness) 3 = Critical (lost time injury, temp. disability) 4 = Catastrophic (serious injury or illness) | 1–11 12–31 32–64 | L – Low M – Medium H – High |

| POTENTIAL HAZARD | POTENTIAL HAZARD DESCRIPTION | RISK ANALYSIS (A x B x C = Degree of Risk) | SUMMARY OF EXISTING CONTROLS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ENGINEERING | ADMINISTRATIVE Alberta’s Occupational Health and Safety Codes (OH&S Code) applicable school jurisdiction administrative policies and regulations | PERSONAL PROTECTIVE EQUIPMENT (PPE) Personal Protective Equipment (PPE) appropriate for the work is both worn and CSA certified/approved where required. | CONTROL IN PLACE YES/NO |
| Biological Exposure | - body fluid exposure from injuries | Biological 4 x 4 x 1 = 16 | - sink for handwashing - first aid kit and supplies | - Use universal precautions. - Ensure that personnel are trained in first aid. - Post emergency procedures. - Identify and establish sanitary and safe housekeeping practices. | - protective gloves (latex or nitrile disposable gloves) - safety glasses, goggles |  YES  NO |
| Chemical Exposure | - exposure to chemicals used in hair styling | Chemical 4 x 4 x 2 = 32 High | - general ventilation and local exhaust | - Ensure WHMIS training is available. - Provide up-to-date MSDS. - Follow safe handling and storage information on labels or MSDS. | - protective gloves (latex or nitrile disposable gloves) |  YES  NO |
|  | - exposure to cleaning and sanitizing chemicals - exposure to residual products | Chemical 4 x 4 x 2 = 32 High | - eyewash station - general ventilation and local exhaust | - Thoroughly remove residual products and chemicals used. - No drinking or eating in class. - Identify and secure materials that are least toxic and harmful to people and the environment. | - protective gloves - safety glasses or goggles (as determined by task severity) |  YES  NO |
| Ergonomic Exposure | - bending and other awkward postures when washing hair | Ergonomic 4 x 4 x 2 = 32 High | - n/a | - Provide and follow instruction on methods to reduce muscle strain when washing hair. - Make adjustments to the chair to reduce time spent in awkward positions. | - n/a |  YES  NO |
|  | - lifting and handling loads, items, equipment, or individuals that are awkward or too heavy - repeated awkward movements or maintaining one position for a long time | Ergonomic 4 x 4 x 2 = 32 High | - n/a | - Perform a hazard assessment, considering the weight, size, shape, and number of times and manner in which the item will be moved. - Get assistance if necessary. - Make sure the trolley load is balanced and that you can see past the load. Push the trolley rather than pull it. - Place heavy objects on lower shelves, between knee level and shoulder level. - Provide non-slip working surfaces where possible. - Develop safe work practices and procedures using required tools, equipment, and materials. | - n/a |  YES  NO |
|  | - frequent, forceful, and repetitive movements | Ergonomic 4 x 4 x 2 = 32 High | - n/a | - Use the appropriate tool for the job. - Take frequent breaks. | - n/a |  YES  NO |
| Physical Exposure | - bodily injury, loss of equipment due to damage | - n/a | - Ensure equipment is installed as per manufacturer’s instructions and safety guidelines. - Provide adequate instruction on lab routines, job procedures and equipment operation; repeat as necessary | - Administrative – maintain manufacturers operating maintenance manuals for program equipment as required by Alberta OHS Adopted Code Part 3 – be familiar with safe operating and maintenance requirements - Preventative maintenance – or reporting defects on appliances - Training in the safe use of machinery and utensils. | - n/a |  YES  NO |
|  | - slippery floors | Physical 4 x 4 x 3 = 48 High | - n/a | - Clean up spills immediately. - Ensure good housekeeping practices. | - n/a |  YES  NO |
|  | - shock and electrocution | Physical 4 x 4 x 2 = 32 High | - n/a | - Maintain portable and fixed dryers in good condition. - Maintain electrical tools and equipment in good condition. - Secure cords with ties to reduce tangles. | - n/a |  YES  NO |
|  | - electrical shock and electrocution from damaged wiring or cords on appliances | Physical 4 x 4 x 3 = 48 High | - main power shut-off switch | - Inspect electrical connections, cords, and outlets for damage. Repair or replace damaged items. | - n/a |  YES  NO |
|  | - fire risk due to damaged or poorly maintained equipment | Physical 4 x 4 x 3 = 48 High | - n/a | - Post evacuation and fire drill procedures. - Use recommended temperature settings for each type of equipment, and shut them off when finished task. - Check cords for damage. | - n/a |  YES  NO |
|  | - insufficient lighting to perform work activities | Physical 4 x 4 x 3 = 48 High | - n/a | - Provide adequate lighting over work areas. | - n/a |  YES  NO |
|  | - falls from heights | Physical 4 x 4 x 3 = 48 High | - n/a | - Stepladders and stools should be used to reach supplies or equipment from storage. Do not climb shelving units. | - n/a |  YES  NO |
| Psycho-social Exposure | - potential for bullying and verbal harassment with adults and students | Psychosocial 1 x 4 x 1 = 4 Low | - n/a | - Follow visitor procedures. - Follow working alone procedures for school staff (including security and door locking procedures). - Follow safe and secure school procedures. | - n/a |  YES  NO |
|  | - fatigue | Psychosocial 1 x 4 x 1 = 4 Low | - n/a | - Schedule regular breaks. - Vary the tasks. - Change the way a task is completed when/where possible. | - n/a |  YES  NO |
|  | - stress | Psychosocial 1 x 4 x 1 = 4 Low | - n/a | - Facilitate conflict management. - Ensure adequate training and time to complete a task. | - n/a |  YES  NO |

© Calgary Board of Education