| NAT: NATURAL RESOURCES |  |  |  |
| --- | --- | --- | --- |
| Wildlife |  |  |  |
| task | Using a Bow |  |  |
| hazards | Potential Hazards |  | Possible Consequences |
| Physical | - lifting and handling loads - repetitive motion - moving parts: struck by string or caught in mechanical release - vibration |  | - death - musculoskeletal injuries - contusions - lacerations - abrasions - pinching - puncture wounds |
| Chemical | - n/a |  | - n/a |
| Biological | - allergens: plants and pollen - bacteria/viruses - blood - body fluids |  | - allergy reactions - anaphylactic shock - illness - infection |
| Ergonomic | - poor or awkward posture - repetitive motion - strength to draw bow - bow not fitted properly to the user |  | - bodily injury |
| Psychological | - fatigue - stress - fear |  | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check for cracks, dents, breaks, separating laminates, peeling glass, and defects in mechanical parts. - Check the bowstring regularly, and replace the bowstring if it becomes worn or frayed. - Use a bow stringer to string a recurve bow. - Report any hazards. - First-time bow hunters should be measured for their draw length. Bows should be fitted to the user. |  |  |  |
| Safe Work Procedure |  | Cautions |  |
| - Put on all required PPE. - Secure loose clothing, and tie back long hair. - Read and follow the manufacturer’s instructions, precautions, and directions. - Assume the shooting position. - Nock the arrow. Only nock an arrow when it’s safe to shoot. - Draw and anchor the bow. - Grip the bow handle or riser, but don’t squeeze. - Present the bow to target. - With bow arm straight, raise the bow until the arm is parallel to the ground while drawing the string back to the anchor point with the shooting hand. - Aim using a “shoot where you look” technique or aim using a sight. If you use a sight, make sure to have the proper sight picture prior to releasing an arrow. - Release the string and follow through. - Put away all tools, equipment, and materials in their proper storage areas. - Clean the work area. - Use proper hygiene. Establish handwashing practices. |  | - Only point the bow and arrow in a safe direction. - Only nock an arrow when it’s safe to shoot. - Be sure of the target and what is in front of the target, immediately behind the target, and beyond the target. - Never shoot an arrow over a ridge. - Only shoot when there is a safe range or shooting area and a safe backstop or background. - Avoid dry-firing a bow. - Do not shoot an arrow straight up in the air. |  |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |  |  |