| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |
| --- | --- | --- |
| Construction |  |  |
| task | Using a Thickness Planer |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - flying objects - electrical - lifting and handling loads - noise - repetitive motion - moving parts - sharp blade - vibration | - bruising - pinching - cuts/amputations - hearing loss - eye irritation and/or eye damage - electrical shock - chronic pain |
| Chemical | - wood dust - human-made material particulates (e.g., lexan, polycarbonate, laminates, MDF, and plywood) | - respiratory damage |
| Biological | - bacteria/virus - blood - body fluids | - illness |
| Ergonomic | - lifting heavy stock materials - lighting | - bodily injury - eye strain |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check for electrical cord and switch damage. - Inspect the material for knots, splits, warp, nails, staples, or other defects. - Inspect PPE and replace if required. - Report any hazards. - Check that all guards are in place and working correctly. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on required PPE. - Secure loose clothing, and tie back hair. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Position the material (with at least one flat face) on the machine infeed table with a flat face down. - Adjust the table or cutter head height, depending on the type of machine. - Turn the machine on. - Feed the material into the infeed side of the planer. - Support long boards until they are halfway through the planer. - Receive the material from the outfeed table. - Readjust the table/cutter head height, and repeat until the desired thickness is achieved. Remove equal amounts of wood from both faces to minimize warp. - Turn the machine off, and wait for the blade to completely stop. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. | - Do not breathe in vapours or dust. - Use a stable and comfortable stance. - Do not reach past the ends of the infeed or outfeed tables or into the machine. - Stock must be longer than the distance between the feed rollers. - Do not look into the planer while it is running. - Stand clear of the ejection path of the material. - Be aware of the grain run-out. Feed the stock so that the cutter head will not tear run-out grain. - Never attempt to plane across the grain; however, planing at an angle to the grain can minimize tear-out. - Jamming or a slow feed rate often indicates dull knives. Sharpen and reset the knives. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |