| Task hazard analysis |  |  |
| --- | --- | --- |
| Electro-Technologies |  |  |
| task | Using a Soldering Station |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - heat | - burns |
| Chemical | - corrosive material and fumes: rosin flux - poisons/toxins: lead, if using a lead-based solder | - dermatitis - lung and/or eye irritation, occupational asthma, and worsening of existing asthmatic conditions - lead poisoning, if ingested, when using a lead-based solder |
| Biological | - bacteria/viruses - blood - body fluids | - Where others have touched devices, possible contamination via viruses/bacteria may occur. |
| Ergonomic | - n/a | - n/a |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. For example, check for electrical cord and solder pencil cord damage. - Inspect PPE and replace if required. - Report any hazards. - Check that the ventilation system is operational before starting to solder. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on all required PPE. - Use tools and equipment under competent supervision. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Add water to station sponge as needed. - Switch on the ventilation equipment. - Switch on the solder station and set the temperature. - Tin the soldering tip and wipe on sponge. - Use the proper soldering sequence: tip to parts, solder to parts, solder away from parts, and tip away from parts. - Wipe the tip on sponge and switch off soldering station. - Put away all tools, equipment, and materials in their proper storage areas. - Clean the work area. Make sure there is no lead solder left on floors or work surfaces. If lead solder is allowed to accumulate on floors and work surfaces, the lead solder can be tracked home. - Establish handwashing practices. Thoroughly wash hands with soap and water to remove any possible flux or lead residue. | - Ensure intakes from the ventilation equipment are close to the work area and functioning properly. Do not breathe vapours. - The soldering tip can cause burns. - If lead solder is allowed to accumulate on floors and work surfaces, it can be tracked home. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |