| TMT: TRADES, MANUFACTURING, and TRANSPORTATION |  |  |
| --- | --- | --- |
| Fabrication |  |  |
| task | Using Electrodes |  |
| hazards | Potential Hazards | Possible Consequences |
| Physical | - lifting and handling loads | - strains - crushing injuries - bruises |
| Chemical | - tungsten non-consumable electrodes are radioactive: thorium oxide | - small external hazard: radiation poisoning - internal hazard: ingestion or inhalation |
| Biological | - bacteria/viruses - blood - body fluids | - illness |
| Ergonomic | - manual handling of consumables - lifting - repetitive work | - bodily injury - chronic pain |
| Psychological | - fatigue - stress | - anxiety - inattentiveness - poor morale - increased illness |
| Pre-operational Safety Checks |  |  |
| - Inspect the work area for hazards. - Inspect tools, equipment, and materials for damage and other hazards. - Inspect PPE and replace if required. - Report any hazards. - Check that the ventilation system is operational before grinding. |  |  |

| Safe Work Procedure | Cautions |
| --- | --- |
| - Put on all required PPE. Note: Inhalation should be avoided when preparing the tip; therefore, a respirator should be mandatory PPE. - Secure loose clothing, and tie back long hair. - Use tools and equipment under competent supervision. - Grinding creates the greatest hazard, as the exposed tungsten/thoria area is greatly increased and fine particles of potentially radioactive dust are released into the atmosphere. Minimize exposure of others to radioactive dust. A respirator should be mandatory PPE. - Use a dedicated grindstone for electrode preparation. - Follow the manufacturer’s instructions. - Follow the school safety policy. - Store thoriated electrodes in steel boxes, clearly labelled with the radiation trefoil. - Put away all tools, equipment, and materials in their proper storage areas. - Keep your work area clean as you work and make sure to clean up when you are done. - Establish handwashing practices. | - Tungsten electrodes are radioactive. Handle with care, and avoid inhalation when preparing the tip. |
| REPORT ANY HAZARDOUS SITUATION IMMEDIATELY! If an emergency occurs while completing any task or the tool or equipment malfunctions, shut off the power immediately, follow the lock-out procedure, and get help. |  |